Suffix code

| Model | Code | Description |
| --- | --- | --- |
| HY-606 |  | 4 tones melody buzzer |
| Power supply voltage | MD | 12 / 24 V d.c |
|  | MA | 110 - 220 V a.c |

· Specification

| Model |  | HY-606 MD, MA |
| --- | --- | --- |
| Appearance |  |  |
| Buzzer sound |  |  |
|  |  | :PLAY :STOP ※ Please press button to listen buzzer |
| Decibel |  | 98 ㏈ Max. |
| Power supply voltage |  | 12, 24 V DC (HY-606MD) / 110, 220 V AC (HY-606MA) |
| Power frequency |  | 50/60 ㎐ (Dual usage) - (HY-606MA) |
| Power consumption |  | Approx 2.5 W |
| Installation hole |  | Ø66 panel cutout |
| Tone types | H1 | Fire |
|  | H2 | Ambulance |
|  | H3 | Police |
|  | H4 | Music (For Elise) |