Suffix code

| Model | Code | Information |  |  |
| --- | --- | --- | --- | --- |
| LF4N- |  | Digital timer 48(W) X 48(H) ㎜ |  |  |
| Device selection | A | Time limit : 1c | Operation mode (10 kinds) | 8pin socket type |
|  | B | Time limit : 1c, Constantaneous : 1c | ON Delay output (A mode fixed) |  |
|  | C | Time limit : 2c |  |  |
|  | D | Time limit : 1c | Operation mode (10 kinds) | 11pin socket type |
| Power supply voltage |  | 24 - 240 V AC / DC dual usage 50 - 60 ㎐ |  |  |

· Specification

| Model |  | LF4N |  |
| --- | --- | --- | --- |
| Power voltage |  | 24 - 240 V a.c / d.c 50 - 60 Hz |  |
| Display |  | LCD display |  |
| Voltage regulation |  | ±10 % from supply voltage |  |
| Power consumption |  | LF4N-A, LF4N-D | 1.5 W(240 V d.c), 4.4 VA(240 V a.c) |
|  |  | LF4N-B, LF4N-C | 2.2 W(240 V d.c), 7.2 VA(240 V a.c) |
| Resetting time |  | Max. 0.1 second |  |
| Minimum Signal Time |  | LF4N-A, LF4N-D | Over 20 ms(START, INHIBIT, RESET) |
| Input Condition | START Input | LF4N-A, LF4N-D | ㆍNon voltage input Impedance in a short circuit : Max. 2 kΩ Residual voltage in a short circuit : Max. 0.7 V d.c Impedance in open : Min. 100 kΩ |
|  | INHIBIT Input |  |  |
|  | RESET Input |  |  |
| Repeatability of operating time |  | LF4N-A, LF4N-D | Max. ±0.01% ±0.05 second(In case of Power-on start) |
|  |  |  | Max. ±0.005% ±0.03 second(In case of Signal-on start) |
|  |  | LF4N-B, LF4N-C | ±0.01% ±0.05 second(In case of Power-on start) |
| Control output |  | Contact relay : 250 V a.c 3 A(Load Resistance) |  |
| Insulation resistance |  | Min 100 MΩ (Based on 500 V d.c) |  |
| Dielectric strength |  | 2000 V a.c 50 / 60 Hz for 1 minute |  |
| Impulse Voltage |  | ±2000 V Max. |  |
| Vibration | Mechanical durability | 10 - 55 Hz double amplitude 0.75 mm |  |
|  | Malfunction durability | 10 - 55 Hz double amplitude 0.5 mm |  |
| Shock | Mechanical durability | 300 m/s² (Less than 30 G) |  |
|  | Malfunction durability | 100 m/s² (Less than 10 G) |  |
| Life | Mechanical durability | Over 1,000 million (Open & Short frequency : 180 / min) |  |
|  | Electrical durability | Over 10 million (250 V a.c 3 A load Resistance) |  |
| Terminal type |  | Plug 8PIN, Plug 11PIN |  |
| Operation ambient temperature |  | -10 ~ 55 °C (No condensation) |  |
| Storage temperature |  | -20 ~ 65 °C (No condensation) |  |
| Operation ambient humidity |  | 35 ~ 85 % R.H. |  |
| Weight |  | About 110 g (Include Fixing bracket) |  |

| Model | LF4N-A | LF4N-B | LF4N-C | LF4N-D |
| --- | --- | --- | --- | --- |
| Power voltage | 24 - 240 V a.c / d.c 50 - 60 Hz |  |  |  |
| Socket | 8 Pin socket |  |  | 11 Pin socket |
| Output | Time limit 1c | Time limit 1c Instantaneous 1c | Time limit 2c | Time limit 1c |
| Timing range | 10 kinds of timing ranges |  |  |  |
| Output operations | 10 kinds | Fixing A mode |  | 10 kinds |
| Input signal | RESET, START, INHIBIT | No Input |  | RESET,START,INHIBIT |