· Suffix code

| Model |  | Suffix code |  |  | Information |
| --- | --- | --- | --- | --- | --- |
| Dimension | - |  |  |  | Analog Timer |
|  | T38N |  |  |  | T38N timer (40 X 50 ㎜) |
|  | T48N |  |  |  | T48N timer (48 X 48 ㎜) |
|  | T57N |  |  |  | T57N timer (58 X 84 ㎜) |
|  | TF62N |  |  |  | TF62N twin timer (58 X 84 ㎜) |
|  | TF62D |  |  |  | TF62D dual timer (58 X 84 ㎜) |
| Installation type |  | E |  |  | Exposure type (Select with T48N panel type) |
|  |  | P |  |  | Panel type (T38N panel adapter sold separately) |
| Range (Deep switch selection) |  |  | 01 |  | 1 sec / 1 min / 1 hour |
|  |  |  | 03 |  | 3 sec / 3 min / 3 hour |
|  |  |  | 06 |  | 6 sec / 6 min / 6 hour |
|  |  |  | 10 |  | 10 sec / 10 min / 10 hour |
|  |  |  | 30 |  | 30 sec / 30 min / 30 hour |
|  |  |  | 60 |  | 60 sec / 60 min / 60 hour |
|  |  |  | 12H |  | 12 hour, 24 hour, 48 hour (※ But exclude the model TF62N and TF62D) |
| Control output |  |  |  | A | Time limit : 1c contact, Constantaneous : 1a contact |
|  |  |  |  | B | Time limit : 1c contact, moment : 1c contact |
|  |  |  |  | C | Time limit : 2 x 1c |
|  |  |  |  | D | TF62N twin timer fixing code |
|  |  |  |  | F | TF62D dual timer fixing code |
| Power supply voltage |  |  |  |  | 24 - 240 V AC / DC 50 - 60 ㎐ (dual usage) |

· Specification

| Model |  | T38N |
| --- | --- | --- |
| Function |  | POWER ON DELAY TIMER |
| Power supply |  | 24 - 240 V a.c 50 / 60Hz, 24 - 240 V d.c ( ±10 %) |
| Allowable voltage variation |  | ± 10 % of Power supply voltage |
| Power Consumption |  | Less than 4.5 VA (at 240 V a.c 60 Hz), Less than 1.5 W (at 24 V d.c) |
| Return time |  | Less than 100 ms |
| maximum time | 01 | 0.01 ~ 1 s / 0.01 ~ 1 m / 0.01 ~ 1 h |
|  | 03 | 0.01 ~ 3 s / 0.01 ~ 3 m / 0.01 ~ 3 h |
|  | 06 | 0.01 ~ 6 s / 0.01 ~ 6 m / 0.01 ~ 6 h |
|  | 10 | 0.01 ~ 10 s / 0.01 ~ 10 m / 0.01 ~ 10 h |
|  | 30 | 0.01 ~ 30 s / 0.01 ~ 30 m / 0.01 ~ 30 h |
|  | 60 | 0.01 ~ 60 s / 0.01 ~ 60 m / 0.01 ~ 60 h |
|  | 12H | 0.01 ~ 12 h / 0.01 ~ 24 h / 0.01~ 48 h (For 24 hours 'x2' and for 48 hours 'x4' ) |
| Time error | Repeat error | Less than 0.3 % (ratio against Max. scale) |
|  | Setting error | Less than 5 % (ratio against Max. scale) |
| Control output | Output mode | Power Time - limit |
|  | Contact | A type(On - delay 1c + Constant contact 1a) / B type(On - delay 1c + Constant contact 1c) / C type(On - delay 2c) |
|  | Contact capacity | 250 V a.c 3 A Resistive load |
| Life span of relay |  | Mechanical : More than 10 million times / Electrical : More than 100,000 times |
| Dielectric strength |  | 2000 V a.c 50 / 60 Hz for 1 minute |
| Noise Immunity |  | ±2 kV (Between unit's power terminals), square wave by noise simulator (pulse width : 1 ㎲) |
| Insulation resistance |  | More than 100 MΩ (Based on 500 V d.c mega standard) |
| Vibration (Durability) |  | 10 - 55 Hz (cycle 1 minute), Double amplitude 0.5 mm X Y Z each direction for 2h |
| Shock (Durability) |  | 300 m/s² (30 G) X Y Z each direction 3 times |
| Ambient temperature |  | - 10 ~ 55 °C Without freeze |
| Storage temperature |  | - 25 ~ 65 °C Without freeze |
| Ambient humidity |  | 30 ~ 85 % R.H. |